МИНИСТЕРСТВО ЦИФРОВОГО РАЗВИТИЯ СВЯЗИ И МАССОВЫХ КОММУНИКАЦИЙ

Ордена Трудового Красного Знамени

Федеральное государственное бюджетное образовательное учреждение высшего образования

«Московский технический университет связи и информатики»

Кафедра «Математическая кибернетика и информационные технологии»

Дисциплина «Проектный практикум»

Отчёт

на тему «Подсистемы светофорного управления»

Выполнила:

Студентка группы БВТ2303

Морозова Ольга

Проверил:

Преподаватель

Мосева Марина Сергеевна

СОДЕРЖАНИЕ

Оглавление

СОДЕРЖАНИЕ	2

ВВЕДЕНИЕ	3

ОБЗОР ПРОДЕЛАННОЙ РАБОТЫ И ЕЁ РЕЗУЛЬТАТЫ	4

ИСПОЛЬЗУЕМЫЕ ТЕХНОЛОГИИ	5

ЗАКЛЮЧЕНИЕ	15

СПИСОК ИСПОЛЬЗУЕМОЙ ЛИТЕРАТУРЫ	15

ВВЕДЕНИЕ

Актуальность данной темы обусловлена несколькими факторами, которые имеют значительное влияние на нынешние транспортные системы:

- Большая интенсивность транспортного потока в наше время;

- Увеличение количества автомобилей на дороге;

- Постоянные пробки и заторы в крупных городах и на основных магистралях.

И для того, чтобы справляться с данной ситуацией и уменьшить загруженность улиц можно использовать “умные” светофоры, которые будут адаптироваться к ситуации на дороге и позволят более рационально распределять время ожидания на перекрёстках, тем самым снижая количество заторов и общую загруженность на некоторых местах дороги.

Целью моей работы было создание нейросети и её обучение с использованием машинного обучения с подкреплением для “умного” светофора.

ОБЗОР ПРОДЕЛАННОЙ РАБОТЫ И ЕЁ РЕЗУЛЬТАТЫ

Результатом моей работы стал проект, созданный при помощи программы SUMO. В нём я создала перекрёсток, настроив возможные направления движения для каждой полосы, а также настроила появление и движение легковых машин. Также для проверки эффективности работы я сделала разное количество машин в час на каждом направлении (1800 и 1300 на “главной” дороге, 900 и 600 на “второстепенной”).

Далее я создала светофор на этом перекрёстке и настроила фазы его работы: две основные, которые дают зелёный свет “главной” и “второстепенной” дорогам соответственно. Именно между ними светофор переключается в своём “стандартном” режиме. Остальные же 4 возможные фазы для переключения нейросетью дают “зелёную дорогу” какому-то одному направлению, помогая его разгрузить.

После написала программу на Python, которая запускала интерактивную эмуляцию SUMO и по ней происходило обучение нейросети.

Также написала программу для использования уже обученной нейросети на эмуляции.

ИСПОЛЬЗУЕМЫЕ ТЕХНОЛОГИИ

В моём проекте я использовала программу SUMO для эмуляции дорожного движения и проверки работоспособности “умного” светофора.

На основе исследуемых статей и проведённых экспериментов я прописала фазы светофора, между которыми переключается нейросеть.

Я использовала машинное обучение с подкреплением для нейросети. Как и для последующего тестового запуска, я использовала для этого программу, написанную на языке Python, и эмуляцию SUMO.

Листинг 1.

Машинное обучение.

# Step 1: Add modules to provide access to specific libraries and functions import os # Module provides functions to handle file paths, directories, environment variables import sys # Module provides access to Python-specific system parameters and functions import random import numpy as np import matplotlib.pyplot as plt # Visualization # Step 1.1: (Additional) Imports for Deep Q-Learning import tensorflow as tf from tensorflow import keras from keras import layers script_dir = os.path.dirname(os.path.abspath(__file__)) os.chdir(script_dir) # Step 2: Establish path to SUMO (SUMO_HOME) if 'SUMO_HOME' in os.environ: tools = os.path.join(os.environ['SUMO_HOME'], 'tools') sys.path.append(tools) else: sys.exit("Please declare environment variable 'SUMO_HOME'") # Step 3: Add Traci module to provide access to specific libraries and functions import traci # Static network information (such as reading and analyzing network files) # Step 4: Define Sumo configuration Sumo_config = [ 'sumo-gui', '-c', 'cros.sumocfg', '--start', '--step-length', '0.10', '--delay', '500', '--lateral-resolution', '0.20' ] # Step 5: Open connection between SUMO and Traci traci.start(Sumo_config) traci.gui.setSchema("View #0", "real world") required_files = ["cros.sumocfg", "cros.net.xml", "cros.rou.xml", "cros.add.xml"] for file in required_files: if not os.path.exists(file): print(f"Ошибка: файл {file} не найден в {script_dir}!") print("Доступные файлы:", os.listdir()) sys.exit(1) # ------------------------- # Step 6: Define Variables # ------------------------- # Variables for RL State (queue lengths from detectors and current phase) q_db_0 = 0 q_db_1 = 0 q_dl_0 = 0 q_dl_1 = 0 q_dl_2 = 0 q_dr_0 = 0 q_dr_1 = 0 q_dr_2 = 0 q_dt_0 = 0 q_dt_1 = 0 current_phase = 0 # ---- Reinforcement Learning Hyperparameters ---- TOTAL_STEPS = 10000 # The total number of simulation steps for continuous (online) training. ALPHA = 0.1 # Learning rate (α) between[0, 1] #If α = 1, you fully replace the old Q-value with the newly computed estimate. #If α = 0, you ignore the new estimate and never update the Q-value. GAMMA = 0.9 # Discount factor (γ) between[0, 1] #If γ = 0, the agent only cares about the reward at the current step (no future rewards). #If γ = 1, the agent cares equally about current and future rewards. EPSILON = 0.1 # Exploration rate (ε) between[0, 1] #If ε = 0 means very greedy, if=1 means very random ACTIONS = [0, 1, 2, 3, 4, 5] # The discrete action space (0 = keep phase, 1 = switch phase on one, switch phase on two, ...) # ---- Additional Stability Parameters ---- MIN_GREEN_STEPS = 100 last_switch_step = -MIN_GREEN_STEPS # ---- Settings for Model Managing ---- MODEL_SAVE = "traffic_light_model.keras" MODEL_LOAD = "traffic_light_model.keras" build_load_switch = 0 # 0 - build / 1 - load # ------------------------- # Step 7: Define Functions # ------------------------- def build_model(state_size, action_size): """ Build a simple feedforward neural network that approximates Q-values. """ model = keras.Sequential() # Feedforward neural network model.add(layers.Input(shape=(state_size,))) # Input layer model.add(layers.Dense(32, activation='relu')) # First hidden layer model.add(layers.Dense(32, activation='relu')) # Second hidden layer model.add(layers.Dense(action_size, activation='linear')) # Output layer model.compile( loss='mse', optimizer=keras.optimizers.Adam(learning_rate=0.001) ) return model def to_array(state_tuple): """ Convert the state tuple into a NumPy array for neural network input. """ return np.array(state_tuple, dtype=np.float32).reshape((1, -1)) def save_model(model, filename=MODEL_SAVE): model.save(filename) print(f"Model was saved: {filename}") def load_model(filename=MODEL_LOAD): if os.path.exists(filename): model = keras.models.load_model(filename) print(f"Model was loaded: {filename}") return model else: print(f"Model file {filename} was not found!") return None # Create the DQN model state_size = 11 # (q_db_0, q_db_1, q_dl_0, q_dl_1, q_dl_2, q_dr_0, q_dr_1, q_dr_2, q_dt_0, q_dt_1, current_phase) action_size = len(ACTIONS) # Building or loading DQN model if build_load_switch == 0: dqn_model = build_model(state_size, action_size) else: dqn_model = load_model() def get_max_Q_value_of_state(s): #1. Objective Function state_array = to_array(s) Q_values = dqn_model.predict(state_array, verbose=0)[0] # shape: (action_size,) return np.max(Q_values) def get_reward(state): #2. Constraint 2 """ Simple reward function: Negative of total queue length to encourage shorter queues. """ total_queue = sum(x ** 2 for x in state[:-1]) # Exclude the current_phase element reward = -float(total_queue) return reward def get_state(): #3&4. Constraint 3 & 4 global q_db_0, q_db_1, q_dl_0, q_dl_1, q_dl_2, q_dr_0, q_dr_1, q_dr_2, q_dt_0, q_dt_1, current_phase # Detector IDs for left detector_dl_0 = "D_l_0" detector_dl_1 = "D_l_1" detector_dl_2 = "D_l_2" # Detector IDs for right detector_dr_0 = "D_r_0" detector_dr_1 = "D_r_1" detector_dr_2 = "D_r_2" # Detector IDs for top detector_dt_0 = "D_t_0" detector_dt_1 = "D_t_1" # Detector IDs for bottom detector_db_0 = "D_b_0" detector_db_1 = "D_b_1" # Traffic light ID traffic_light_id = "tl" # Get queue lengths from each detector q_dl_0 = get_queue_length(detector_dl_0) q_dl_1 = get_queue_length(detector_dl_1) q_dl_2 = get_queue_length(detector_dl_2) q_dr_0 = get_queue_length(detector_dr_0) q_dr_1 = get_queue_length(detector_dr_1) q_dr_2 = get_queue_length(detector_dr_2) q_dt_0 = get_queue_length(detector_dt_0) q_dt_1 = get_queue_length(detector_dt_1) q_db_0 = get_queue_length(detector_db_0) q_db_1 = get_queue_length(detector_db_1) # Get current phase index current_phase = get_current_phase(traffic_light_id) return (q_db_0, q_db_1, q_dl_0, q_dl_1, q_dl_2, q_dr_0, q_dr_1, q_dr_2, q_dt_0, q_dt_1, current_phase) def apply_action(action, tls_id="tl"): #5. Constraint 5 """ Executes the chosen action on the traffic light, combining: - Min Green Time check - Switching to the next phase if allowed Constraint #5: Ensure at least MIN_GREEN_STEPS pass before switching again. """ global last_switch_step if action == 0: # Do nothing (keep current phase) return # Check if minimum green time has passed before switching elif current_simulation_step - last_switch_step >= MIN_GREEN_STEPS: program = traci.trafficlight.getAllProgramLogics(tls_id)[0] num_phases = len(program.phases) next_phase = (get_current_phase(tls_id) + action) % num_phases traci.trafficlight.setPhase(tls_id, next_phase) # Record when the switch happened last_switch_step = current_simulation_step def update_Q_table(old_state, action, reward, new_state): #6. Constraint 6 """ In DQN, we do a single-step gradient update instead of a table update. """ # 1) Predict current Q-values from old_state (current state) old_state_array = to_array(old_state) Q_values_old = dqn_model.predict(old_state_array, verbose=0)[0] # 2) Predict Q-values for new_state to get max future Q (new state) new_state_array = to_array(new_state) Q_values_new = dqn_model.predict(new_state_array, verbose=0)[0] best_future_q = np.max(Q_values_new) # 3) Incorporate ALPHA to partially update the Q-value Q_values_old[action] = Q_values_old[action] + ALPHA * (reward + GAMMA * best_future_q - Q_values_old[action]) # 4) Train (fit) the DQN on this single sample dqn_model.fit(old_state_array, np.array([Q_values_old]), verbose=0) def get_action_from_policy(state): #7. Constraint 7 """ Epsilon-greedy strategy using the DQN's predicted Q-values. """ if random.random() < EPSILON: return random.choice(ACTIONS) else: state_array = to_array(state) Q_values = dqn_model.predict(state_array, verbose=0)[0] return int(np.argmax(Q_values)) def get_queue_length(detector_id): #8.Constraint 8 return traci.lanearea.getLastStepVehicleNumber(detector_id) def get_current_phase(tls_id): #8.Constraint 8 return traci.trafficlight.getPhase(tls_id) # ------------------------- # Step 8: Fully Online Continuous Learning Loop # ------------------------- # Lists to record data for plotting step_history = [] reward_history = [] queue_history = [] cumulative_reward = 0.0 print("\n=== Starting Fully Online Continuous Learning (DQN) ===") for step in range(TOTAL_STEPS): current_simulation_step = step # keep this variable for apply_action usage state = get_state() action = get_action_from_policy(state) apply_action(action) traci.simulationStep() # Advance simulation by one step new_state = get_state() reward = get_reward(new_state) cumulative_reward += reward update_Q_table(state, action, reward, new_state) # Print Q-values for the old_state right after update updated_q_vals = dqn_model.predict(to_array(state), verbose=0)[0] # Record data every 100 steps if step % 1 == 0: updated_q_vals = dqn_model.predict(to_array(state), verbose=0)[0] print(f"Step {step}, Current_State: {state}, Action: {action}, New_State: {new_state}, \ Reward: {reward:.2f}, Cumulative Reward: {cumulative_reward:.2f}, Q-values(current_state): {updated_q_vals}") step_history.append(step) reward_history.append(cumulative_reward) queue_history.append(sum(new_state[:-1])) # sum of queue lengths # ------------------------- # Step 9: Close connection between SUMO and Traci # ------------------------- # Also saving out model save_model(dqn_model) traci.close() # ~~~ Print final model summary (replacing Q-table info) ~~~ print("\nOnline Training completed.") print("DQN Model Summary:") dqn_model.summary() # ------------------------- # Visualization of Results # ------------------------- # Plot Cumulative Reward over Simulation Steps plt.figure(figsize=(10, 6)) plt.plot(step_history, reward_history, marker='o', linestyle='-', label="Cumulative Reward") plt.xlabel("Simulation Step") plt.ylabel("Cumulative Reward") plt.title("RL Training (DQN): Cumulative Reward over Steps") plt.legend() plt.grid(True) plt.show() # Plot Total Queue Length over Simulation Steps plt.figure(figsize=(10, 6)) plt.plot(step_history, queue_history, marker='o', linestyle='-', label="Total Queue Length") plt.xlabel("Simulation Step") plt.ylabel("Total Queue Length") plt.title("RL Training (DQN): Queue Length over Steps") plt.legend() plt.grid(True) plt.show()

Листинг 2.

Использование обученной нейросети.

import os import sys import numpy as np import tensorflow as tf from tensorflow import keras from keras import layers import traci script_dir = os.path.dirname(os.path.abspath(__file__)) os.chdir(script_dir) if 'SUMO_HOME' in os.environ: tools = os.path.join(os.environ['SUMO_HOME'], 'tools') sys.path.append(tools) else: sys.exit("Please declare environment variable 'SUMO_HOME'") Sumo_config = [ 'sumo-gui', '-c', 'cros.sumocfg', '--start', '--step-length', '0.10', '--delay', '500', '--lateral-resolution', '0.20' ] traci.start(Sumo_config) traci.gui.setSchema("View #0", "real world") required_files = ["cros.sumocfg", "cros.net.xml", "cros.rou.xml", "cros.add.xml"] for file in required_files: if not os.path.exists(file): print(f"Ошибка: файл {file} не найден в {script_dir}!") print("Доступные файлы:", os.listdir()) sys.exit(1) q_db_0 = 0 q_db_1 = 0 q_dl_0 = 0 q_dl_1 = 0 q_dl_2 = 0 q_dr_0 = 0 q_dr_1 = 0 q_dr_2 = 0 q_dt_0 = 0 q_dt_1 = 0 current_phase = 0 TOTAL_STEPS = 3000 ACTIONS = [0, 1, 2, 3, 4, 5] # The discrete action space (0 = keep phase, 1 = switch phase on one, switch phase on two, ...) MIN_GREEN_STEPS = 100 last_switch_step = -MIN_GREEN_STEPS MODEL_LOAD = "traffic_light_model.keras" def to_array(state_tuple): """ Convert the state tuple into a NumPy array for neural network input. """ return np.array(state_tuple, dtype=np.float32).reshape((1, -1)) def load_model(filename=MODEL_LOAD): if os.path.exists(filename): model = keras.models.load_model(filename) print(f"Model was loaded: {filename}") return model else: print(f"Model file {filename} was not found!") return None dqn_model = load_model() def get_state(): global q_db_0, q_db_1, q_dl_0, q_dl_1, q_dl_2, q_dr_0, q_dr_1, q_dr_2, q_dt_0, q_dt_1, current_phase # Detector IDs for left detector_dl_0 = "D_l_0" detector_dl_1 = "D_l_1" detector_dl_2 = "D_l_2" # Detector IDs for right detector_dr_0 = "D_r_0" detector_dr_1 = "D_r_1" detector_dr_2 = "D_r_2" # Detector IDs for top detector_dt_0 = "D_t_0" detector_dt_1 = "D_t_1" # Detector IDs for bottom detector_db_0 = "D_b_0" detector_db_1 = "D_b_1" # Traffic light ID traffic_light_id = "tl" # Get queue lengths from each detector q_dl_0 = get_queue_length(detector_dl_0) q_dl_1 = get_queue_length(detector_dl_1) q_dl_2 = get_queue_length(detector_dl_2) q_dr_0 = get_queue_length(detector_dr_0) q_dr_1 = get_queue_length(detector_dr_1) q_dr_2 = get_queue_length(detector_dr_2) q_dt_0 = get_queue_length(detector_dt_0) q_dt_1 = get_queue_length(detector_dt_1) q_db_0 = get_queue_length(detector_db_0) q_db_1 = get_queue_length(detector_db_1) # Get current phase index current_phase = get_current_phase(traffic_light_id) return (q_db_0, q_db_1, q_dl_0, q_dl_1, q_dl_2, q_dr_0, q_dr_1, q_dr_2, q_dt_0, q_dt_1, current_phase) def apply_action(action, tls_id="tl"): global last_switch_step if action == 0: # Do nothing (keep current phase) return else: program = traci.trafficlight.getAllProgramLogics(tls_id)[0] num_phases = len(program.phases) next_phase = (get_current_phase(tls_id) + action) % num_phases traci.trafficlight.setPhase(tls_id, next_phase) # Record when the switch happened last_switch_step = current_simulation_step def get_action_from_policy(state, step): if ((step % MIN_GREEN_STEPS != 0)): return 0 else: state_array = to_array(state) Q_values = dqn_model.predict(state_array, verbose=0)[0] return int(np.argmax(Q_values)) def get_queue_length(detector_id): return traci.lanearea.getLastStepVehicleNumber(detector_id) def get_current_phase(tls_id): return traci.trafficlight.getPhase(tls_id) print("\n=== Starting Simulation of Smart Traffic Lights ===") for step in range(TOTAL_STEPS): current_simulation_step = step # keep this variable for apply_action usage state = get_state() action = get_action_from_policy(state, step) apply_action(action) traci.simulationStep() # Advance simulation by one step new_state = get_state() traci.close() print("\nOnline Simulation completed.")

ЗАКЛЮЧЕНИЕ

Подводя итоги проделанной работы, могу сказать, что результат оказался удовлетворительным, ведь написанная и обученная мною нейросеть показала свою работоспособность и эффективность в эмуляции.

СПИСОК ИСПОЛЬЗУЕМОЙ ЛИТЕРАТУРЫ

- Математическое моделирование транспортных потоков на примере управления светофорами (https://official.satbayev.university/download/document/31966/%D0%9D%D0%B0%D0%B3%D0%B0%D0%B9%20%D0%90%D0%BD%D0%B0%D1%81%D1%82%D0%B0%D1%81%D0%B8%D1%8F%20%D0%94%D0%98%D0%9F%D0%9B%D0%9E%D0%9C%202023.pdf);

- Applying Reinforcement Learning to Optimize Traffic Light Cycles;

- Training and Applying Artificial Neural Networks in Traffic Light Control: Improving the Management and Safety of Road Traffic in Tyumen (Russia);

- Deep Reinforcement Learning for the Joint Control of Traffic Light Signaling and Vehicle Speed Advice.